Spatial Resolution

- Determined by how sampling was carried out

- Pixel size, dots per inch (DPI)

- Smaller image size > blur

Intensity / Gray Level Resolution

- Number of intensity levels used to represent the image

- More intensity levels used, finer the level of detail

- Number of bits used to store each intensity level (eg. 8 bits > 2^8 no. of intensity lv.)

- Lower grey levels, lesser choice of colours used in image

Image Enhancement

- Make image more visually appealing (Gamma correction > boost dark values > brighter)

- Highlight interesting detail in images

- Remove noise from images (Median filter > Remove Salt and Pepper Noise)

CMYK: In its color space, the brightness and chroma are distributed over each of the three components

Image reverse operation: NOT applied for enhancing images with mainly brighter grayscale

Image Enhancement: Spatial Domain (ch 2)

- Point processing

  - Gray values change without any knowledge of its surroundings (Grey level transformation)

    - Log (spectrum)

    - Power-law

    - Piecewise linear

      - Contrast stretching

        - Increase the contrast

      - Intensity level slicing

        - Highlighting

    - Binaryzation

    - Histogram Equalization (pdf -> cdf -> transformation)

      - Not for smoothing

      - After point processing, the histogram of the processed image is undeterminable when compared with the original one

      - The corresponding relations between an image and its gray histogram is many to one

- Neighbourhood processing (filtering)

  - Gray values change depending on the grey values in a small neighbourhood of pixels around the given pixel

    - Basic spatial filters

      - Linear spatial filters (see 2B p.4-6)

    - Smoothing filters (remove fine details, blurring and noise reduction)

      - (Neighbourhood) Averaging filters (linear)

      - Weighted smoothing filters

      - Order Statistics filters (non-linear) (Go to page 10)

        - Median filters (better than average filter in removing noise, see 2B p.16)

        - Max filters

        - Min filters

    - Sharpening filters (highlight fine details, remove blurring and highlight edges > spatial differentiation)

      - Laplacian filters (Original – Laplacian = Sharpened)

      - Sobel filters (Gradient mask)

Power Law Transformation

Gamma < 1 : brighter, less dark input values, expansion of lower levels

Gamma > 1 : darker, compression of higher gray levels

*Will not affect the image to become blurred or details look much clear*

Dark > low side of grey scale		Light > high side of grey scale

Low contrast > narrow and centred towards the middle of grey scale	High contrast > broad range of grey scale (evenly spaced histogram)

Image Enhancement: Frequency Domain (ch 3)

Fourier Transform Spectrum (see 3A p.24-28)

- Low freq.: Center of spectrum, Uniform grey values

- High freq.: Boundary of spectrum, Edge and shape transitions in grey values (Small image details and shape edges), NOT responsible for overall gray level display in smooth areas

- After image translation, the amplitude and phase characteristics of the Fourier transform are NOT unchanged

- Features of Fourier Transform:

  - Original data can be fully recovered from the result of transform

  - Optimal in a mean square error sense

  - Has the plural operation

  - Does not have the concept of frequency domain

Filtering in Frequency Domain

- Ideal filtering

  - Has ringing effect in images

- Low-Pass filtering (Ideal / Butterworth / Gaussian)

  - Remove high freq.

  - Remove noise

  - Blur image  Achieve smoothing

  - Butterworth better than ideal (reduce “ringing” while keeping clear cut-off)

  - Gaussian (No ringing but allows high freq. to pass)

- High-Pass filtering (Ideal / Butterworth / Gaussian)

  - Remove low freq.

  - Sharpening, edge enhancement

  - Cannot denoise

- Laplacian filtering

  - Sharpening, better than Laplacian in Spatial

- Homomorphic filtering (see 3B p.36-40)

  - Illumination: decrease contribution made by low freq.

  - Reflection: amplify contribution made by high freq.

- Selective filtering

  - Bandpass / Band-reject

    - Process specific bands

    - Band-reject: remove periodic noise

  - Notch filters

    - Process small regions of the freq. rectangle

    - Rejects / passes specific frequencies

    - Remove periodic noise (eg. image mosaics, scan line noise, halftoning noise)

Morphological Processing (ch 4)

Morphology

- Extract image components

- Applications:

Structuring Elements

- Box-shaped SE

  - Preserve sharp object corners

- Disk-shaped SE

  - Round the corners of the objects

Basic Morphological Methods

- Fit (ALL of the ‘1’s in SE is covered by image)

  - Erosion (applying fit to entire image) (operator: Θ)

    - Contrasts an object

- Hit (any ONE of the ‘1’s in SE is covered by image)

  - Dilation (applying hit to entire image) (operator: ⊕)

    - Expands an object

    - Remember to reflect the SE (along x then along y) before applying dilation

- Opening (Erosion followed by Dilation)

  - Smoothens contours

  - Enlarges narrow gaps

  - Eliminates thin protrusions and ridges

  - Cannot remove holes

  - “Smoothing from inside”

- Closing (Dilation followed by Erosion)

  - Fills narrow gaps, holes and small breaks

  - Preserves objects’ size

  - “Smoothing from outside”

*Bigger the SE, bigger the effect in the image

*Combination of opening and closing: Image with fingerprints

Morphological Algorithms

- Hit or Miss Transform

  - Detecting specific shapes

- Boundary Extraction

- Hole Filling

- Connected Components (similar to hole filling)

- Skeletons

Image Restoration (ch 6)

- Degradation & Restoration Process Models

  - If the image is degraded, it should first make a restoration process, further enhancement

  - The degradation caused by blurring will NOT decrease the spatial resolution of the image

  - Degradation function & Noise function (principle)

- Noise Models

  - Source of Noise

    - Digital image acquisition process

    - Image transmission

  - Models

    - Gaussian

      - Electronic circuit noise, sensor noise (poor illumination / high temperature)

    - Rayleigh

      - Model noise in range imaging

    - Erlang (Gamma)

      - Laser imaging

    - Exponential

      - Laser imaging

    - Uniform

    - Impulse (Salt and Pepper)

      - Find in quick transients (eg. faulty switches)

- Restoration in the presence of noise only (spatial filtering)

  - Mean filters

    - Arithmetic Mean

    - Geometric Mean

      - Multiply all values in the filter ^ (1/MN)

      - Ineffective in the presence of pepper noise

      - Lose less detail than arithmetic

    - Harmonic Mean

      - MN / summation of all (1 / x)

      - Works well for salt noise, Gaussian noise

      - Retains detail better than arithmetic

    - Contraharmonic Mean

      - Sum of x^Q+1 / Sum of x^Q

      - + Q: eliminating pepper noise

      - - Q: eliminating salt noise

  - Order-statistics filters

    - Order filter

      - Arranges all pixels in sequential order based on grey level value in ascending order

    - Minimum filter

      - Works best for salt-type noise (high value)

      - Replace the centre value with the smallest value around it

    - Maximum filter

      - Works best for pepper-type noise (low value)

      - Replace the centre value with the largest value around it

    - Median filter

      - Works best for both salt and pepper noise (high and low values)

      - Better noise remover than Averaging filter

      - Preserve line structures

      - Replace the centre value with the median value around it

    - Midpoint filter

      - Works best for Gaussian and uniform noise

      - Not good for handling salt and pepper noise

      - Replace the centre value with (min + max) / 2

    - Alpha-trimmed mean filter

      - Works best for combination noise such as salt and pepper with Gaussian noise

      - Remove d / 2 highest and lowest values, then sum the remaining values / (MN – d)

  - Adaptive mean filters

    - Depends on the characteristics of image inside the filter region

    - Superior performance but increase in filter complexity

    - Preserve edges

- Estimation of the degradation function

  - Image observation

  - Experimentation

  - Mathematical modelling

    - Atmospheric blur

    - Motion blur

      - Taking images when your hand is not stable

- Inverse filtering

  - Numerical problem

    - If any points in H(u, v) are zero (division by zero)

      - Solutions: Do not take zero points of H(u, v) into account

    - In practice the denominator H(u, v) may have too small magnitude in the high frequency part, making the inverse filter highly unstable.

      - Solutions: To prevent this, we can (1) confine the inverse filter operation only to the low frequency part; or (2) add a small constant in the denominator of the inverse filter process, similar to Wiener filter.

- Wiener filtering (Minimum Mean Square Error filtering)

  - Objectives: Find an estimate of the uncorrupted image such that the mean square error between them is minimum

  - If the noise is zero, the Wiener filter reduces to the inverse filter

  - Noise power spectrum is constant when dealing with spectrally white noise

  - Differences: When the image is blurred by a known lowpass filter, it is possible to recover the image by inverse filtering. However, inverse filtering is very sensitive to additive noise. The approach of reducing one degradation at a time allows us to develop a restoration algorithm for each type of degradation and simply combine them. The Wiener filtering executes an optimal tradeoff between inverse filtering and noise smoothing. It removes the additive noise and inverts the blurring simultaneously.

Segmentation – Categories (ch 8)

Segmentation

- Split/separate/subdivide an image into regions or objects

- To facilitate recognition, understand region of interest

- Challenges:

  - The definition of a region/object is problem-dependent

  - One of the most difficult tasks in image processing

  - Accuracy determines success or failure of application

- Edge-based

- Finding boundary between adjacent regions

- Finding discontinuities (sharp, local changes in intensity)

- Discontinuities in digital images (Spatial filters)

  - Point (Laplacian)

  - Line (horizontal, -45 degree, vertical, 45 degree)

  - Edges (Roberts, Prewitt, Sobel, LOG, Canny)

- Techniques:

  - Point Detection

    - Detect an isolated point

    - Laplacian filter

      - 1st order derivatives

        - Produce thicker edges

      - 2nd order derivatives

        - Stronger response to fine details (thin lines, isolated points, noise)

        - Produce a double-edge response at ramp transitions in intensity

        - Sign: Determine whether a transition into an edge is from light to dark or dark to light

  - Line Detection

    - If each mask is moved around an image, it would respond more strongly to lines in the mentioned direction

    - 2nd derivatives have stronger response and produce thinner lines than 1st derivatives

  - Edge (pixel) detection (see ch8 p.15)

    - Edges: The boundary between two regions with relatively distinct gray-level properties

    - 3 fundamental steps (p.20)

      - Image smoothing for noise reduction

      - Detection of edge points

      - Edge localization

    - Roberts (cross gradient operators)

    - Prewitt & Sobel operators (horizontal, vertical and two diagonal masks)

    - Sobel masks (better noise suppression -> smoothing)

    - Problems of edge detection (see p.29)

    - Reason of not using Laplacian to detect edges (see p.30)

  - Edge formation from edge pixels

    - Edge Linking

      - Canny Filter

        - Use double thresholding and connectivity analysis to detect and link edges

    - Hough Transform

      - Motivation: Isolated points, identifying the locations and orientations of certain types of features in a digital image

      - Problem:

        - “a” approaches infinity as the line gets perpendicular to the x axis. (if a line is perpendicular to x axis, then this line is represented x=M. a->infinity)

      - Solution:

- Threshold-based

- Finding regions by grouping pixels of similar gray values

- Region-based

- Finding regions directly using growing or splitting

- Motion-based

- Finding regions by comparing successive frames of a video sequence to identify regions that correspond to moving objects

Threshold & Region-based Segmentation (ch 9)

  - Threshold-based segmentation

  - Basic Global Thresholding

    - Disadvantage: requires successive calculation of means (for both groups of pixels) at each step (refer ch9 p.19)

  - Optimum Global Thresholding (OTSU)

    - View thresholding as a statistical-decision theory problem

    - Objectives: Maximizes the between-class variance and minimize the with-in class variance

  - Multiple Thresholding

  - Variable Thresholding

  - Region-based segmentation

  - Region grow (refer ch9 p.44)

  - K-means

        - Partition the data points into K clusters randomly and find centroids of each cluster

        - Advantage:

        - Easy for implementation and high-speed performance

        - Measurable and efficient in large data collection

        - Disadvantage:

        - Selection of optimal number of clusters is difficult

        - Selection of the initial centroids is random and different initializations lead to different results

Face Recognition (ch 10 & 11)

- Principal Component Analysis (PCA)

- Converts a set of observations of possibly correlated variables into a set of values of linearly uncorrelated variables called principal components

- Purpose: Identify the orientation with largest variance

- Linear Discriminant Analysis (LDA)

- Eigenfaces exploit the max scatter of the training images in face space

- Fisherfaces attempt to maximize the between class scatter, while minimizing the within class scatter

- Goal: Find the best separation between two classes

- The Eigenfaces Algorithm

  - Pros and Cons (see ch10 p.41)

Deep learning in image segmentation

- Models

  - Fully Convolutional Network, DeconvNet, SegNet, U-Net, PSPNet, DeepLab v1, v2, v3, v3+

- Loss functions

  - Cross entropy (CE),Weighted cross entropy, Balanced cross entropy (BCE), Focal loss, Dice loss, Tversky loss

- Able to choose suitable loss functions

Detection (ch 12)

- HOG

- DPM

- RCNN, Fast-RCNN, Faster-RCNN, Mask-RCNN